| TASK | COSHH Assessment |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| LOCATION OF USE | Workshop |  | #### Assessment No |  | 022 |
| PRODUCT NAME | Dry Moly Spray |  |  |  |  |
| DESCRIPTION OF SUBSTANCE | Aerosol lubricant which protects at high temperatures and high load pressures. (Dark Grey) |  |  |  |  |
| MANUFACTURER | Solent Lubricants |  |  |  |  |
| #### MANUFACTURERS CONTACT | Solent, Wigston, Leicester, L18 2FS Tel: 0116 288 8000 Fax: 0116 288 8222 Emergency Telephone Number: 01942 713667 |  |  |  |  |
| #### ASSESSOR | Martin Monaghan CMIOSH |  | ##### Date |  | 04 12 25 |
| TITLE | Health and Safety Consultant |  | #### Review by |  | #### 04 12 27 |
| Personnel Performing Task: Technicians/ Fitters/ Workshop Operatives |  | Time Spent (Minutes/day) As and when required, amounting to a few minutes per day | ###### Training Given |  |  |
|  |  |  | Yes |  |  |

| MSDS Revision Date | Risk Phrases/Hazard Statements |
| --- | --- |
| May 2015 | H220-Extremely Flammable gas H222- Extremely Flammable Aerosol H226-Flammable liquid and vapour H229- Pressurised container; May burst if heated H280-Contains gas under pressure may explode if heated H304-May be fatal if swallowed and enters airways H319- Causes serious eye irritation H336- May cause drowsiness or dizziness H412- Harmful to aquatic life with lasting effects |

Assessment to be conducted in association with Safety Data Sheets, a full current copy of which must be attached to this assessment.

Please denote the hazardous properties of the substances used in the task with a

NEW INTERNATIONAL GLOBALLY HARMONISED HAZARD SYMBOLS (labelling information)

| Flammable Extremely | Toxic | Harmful Mild irritation on the eye | Oxidising | Corrosive | Explosive | Harmful to the environment | Health Hazards i.e. respiratory sensitisation or carcinogenicity |
| --- | --- | --- | --- | --- | --- | --- | --- |

SMOKING, EATING AND DRINKING ARE PROHIBITED IN THE WORK AREAS.

Information, Instruction and Training must be provided to all users. Young Persons, New or Expectant Mothers and other vulnerable groups must not use or be exposed to this product.

WORKPLACE EXPOSURE LIMITS (WEL’s)

Note: Workplace Exposure Limits are Occupational Exposure Limits set under COSHH Regulations 2004. Harmful substances and their WELs are listed in a publication called EH40 Workplace Exposure Limits. Ingredients with limit values that require monitoring at the workplace: 115-1016 dimethyl ether Short-term value: 958mg/m³, 500ppm WEL Long-term value 766mg/m³, 400ppm 67-64-1 propan-2-one Short-term value: 3620mg/m³, 1500ppm WEL Long-term value 1210mg/m³, 500ppm 1330-20-7 Xylene (mix) Short-term value: 441mg/m³, 100ppm WEL Long-term value 220mg/m³, 50ppm Exposure monitoring must be undertaken to determine accurate worker exposures. Ingredients with biological limit values: 1330-20-7 Xylene (mix) BMGV 650 mmol/mol creatinine Medium:urine Sampling time: post shift Parameter: methyl hippuric acid IMPORTANT! Refer to Workplace Exposure Levels information provided in the Manufacturer’s Safety Data Sheet and revise control measures if exposure levels likely to be reached.

| ROUTE OF EXPOSURE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Skin |  | Eyes |  | Inhalation |  |
| Ingestion |  | Cuts/abrasions |  | Systemic |  |

| PERSONS AT RISK |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| User of product |  | Co workers |  | Members of the public |  |

| ROUTE OF EXPOSURE | FIRST AID MEASURES |
| --- | --- |
| Eyes | Hold eye open and rinse slowly and gently with water for 15- 20 minutes. Obtain medical attention |
| Skin | Wash skin immediately with soap and water. Remove any contaminated clothing. Do not keep contaminated cloths or rags in overall pockets next to the skin. |
| Inhalation | Remove person to fresh air and keep comfortable for breathing. Call a doctor if you feel unwell. |
| Ingestion | Unlikely route of exposure. Wash out mouth with water. Do not induce vomiting. Risk of aspiration |
| In all cases of doubt, or if symptoms persist seek medical attention. Never give anything by mouth to an unconscious person. If unconscious, place in recovery position and seek medical advice. |  |

| PERSONAL PROTECTIVE EQUIPMENT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Face mask/ Dust mask | Respirator |  | Face Shield |
| Tightly sealed goggles | Suitable long-sleeved coveralls. |  | In case of insufficient ventilation, wear suitable respiratory equipment. A suitable mask with filter type A (EN14387 or EN405) may be appropriate. | Blue nitrile rubber gloves or equivalent recommended. |  |

| HANDLING | STORAGE | TRANSPORT | DISPOSAL |
| --- | --- | --- | --- |
| Keep away from heat, hot surfaces, sparks, open flames and other ignition sources. No smoking. Keep away from: Electrostatic charge. Do not spray on an open flame or other ignition source. Do not pierce or burn, even after use. Do not let the can touch battery terminals, electrical connectors or any source of electricity. Use product in a well-ventilated area only. Ensure adequate ventilation. Avoid breathing vapours. Avoid prolonged skin contact. Wear protective gloves. | Protect from sunlight. Store in a well-ventilated place. Industrial product for professional use. Locked storage does not apply to this product. Storage Temperature Do not expose to temperatures exceeding 50°C/ 122°F. Storage Life Stable under normal conditions | Keep aerosols upright, do not mix with other chemicals. Incompatible materials: Oxidizing agents, Reducing agent. | Must not be disposed together with household garbage. Do not allow product to reach sewage system. Disposal must be made according to official regulations. |

GENERAL CONTROL MEASURES

- Users to avoid contact with skin and eyes, and inhalation of vapours/ spray or mist arising from the application of this preparation – Use PPE.

- Only opened solvent containers should be kept at the bench.

- When not in use, opened solvent containers should be kept in the flammables cupboards, under the work benches.

- Surplus/ un-opened solvents must be kept in stores/ parts department.

- Ensure adequate welfare and washing facilities available and provide users with barrier and restorative creams.

- Ensure operatives are made aware of and understand the purpose of this assessment and SDS, plus any health implications of using the product.

- Ensure COSHH assessments with accompanying SDS are made available for all employees and users and for inspection and emergency purpose.

- Carry out personal exposure monitoring.

FIRE PRECAUTIONS

Suitable Fire Extinguishing Methods: CO2, powder or water spray. Fight fires with water spray or alcohol resistant foam Special hazards arising from the substance or mixture Heating may cause pressure rise with risk of bursting. Advice for fire-fighters Fire fighters should wear complete protective clothing including self-contained breathing apparatus. If it is safe to do so, containers should be removed from fire area because they are likely to rupture under fire conditions. Keep fire exposed containers cool by spraying with water.

ACCIDENTAL RELEASE/SPILLAGE PROCEDURES

The product is an aerosol. It is unlikely to present spillage or leakage hazard. In case of rupture, released content should be contained as any other solvent spill. Personal precautions, protective equipment and emergency procedures Wear protective equipment. Keep unprotected persons away. Environmental precautions Do not allow to enter drains, sewers or watercourses. Spillages or uncontrolled discharges into watercourses must be alerted to the appropriate regulatory body. Methods and material for containment and cleaning up Place leaking cans in a bucket or pail to minimise product contamination of surfaces. Collect spillage. Absorb remaining liquid in sand or inert absorbent and remove to safe place. Transfer to a container for disposal. Do NOT wash away into sewer. Containers must not be punctured or destroyed by burning, even when empty.

MONITORING REQUIREMENTS AND HEALTH SURVEILLANCE

No health surveillance in place at present.