| TASK | Thinners mixed to paint spray and wiping down surfaces |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| LOCATION OF USE | Workshop |  | #### Assessment No |  | 028 |
| PRODUCT NAME |  |  |  |  |  |
| DESCRIPTION OF SUBSTANCE |  |  |  |  |  |
| MANUFACTURER | Thomas Howse Ltd |  |  |  |  |
| #### MANUFACTURERS CONTACT | Cakemore Road, Rowley Regis, West Midlands, B65 0RD 07736610239 |  |  |  |  |
| #### ASSESSOR | Martin Monaghan CMIOSH |  | ##### Date |  | 04 12 25 |
| TITLE | Health and Safety Consultant |  | #### Review by |  | #### 04 12 27 |
| Personnel Performing Task (Please list below) Workshop Operatives |  | Time Spent (Minutes/day) As and when required, amounting to a few minutes per day | ###### Training Given |  |  |
|  |  |  | Yes |  |  |

| MSDS Revision Date | Hazard Statement |
| --- | --- |
| 18/01/2019 Version 2 | Flammable liquid and vapour Harmful if inhaled Causes skin irritation Causes skin irritation May be fatal if swallowed and enters airways Risk of explosion if heated under confinement |

Assessment to be conducted in Association with Hazard Data Sheets, a full current copy of which must be attached to this assessment.

Please denote the hazardous properties of the substances used in the task with a

NEW INTERNATIONAL GLOBALLY HARMONISED HAZARD SYMBOLS

| Flammable | Toxic | Warning | Oxidising | Corrosive | Explosive | Harmful to the environment | Health Hazards i.e. respiratory sensitisation or carcinogenicity |
| --- | --- | --- | --- | --- | --- | --- | --- |

SMOKING, EATING AND DRINKING ARE PROHIBITED IN THE WORK AREAS.

Information, Instruction and Training must be provided to all users. Young Persons, New or Expectant Mothers and other vulnerable groups must not use or be exposed to this product.

WORKPLACE EXPOSURE LIMITS (WEL’s)

IMPORTANT! Refer to Workplace Exposure Levels information provided in the Manufacturers Safety Data and revise control measures if exposure levels likely to be reached.

| ROUTE OF EXPOSURE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Skin |  | Eyes |  | Inhalation |  |
| Ingestion |  | Cuts/abrasions |  | Systemic |  |

| PERSONS AT RISK |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| User of product |  | Co workers |  | Members of the public |  |

| ROUTE OF EXPOSURE | FIRST AID MEASURES |
| --- | --- |
| Eyes | Immediately flush eyes with plenty of water, occasionally lifting the upper and lower eyelids. Check for and remove any contact lenses. Continue to rinse for at least 10 minutes. Get medical attention. |
| Skin | Wash with plenty of soap and water. Remove contaminated clothing and shoes. Wash contaminated clothing thoroughly with water before removing it, or wear gloves. Continue to rinse for at least 10 minutes. Get medical attention. If necessary, call a Wash with plenty of soap and water. Remove contaminated clothing and shoes. |
| Inhalation | Remove victim to fresh air and keep at rest in a position comfortable for breathing. If it is suspected that fumes are still present, the rescuer should wear an appropriate mask or self-contained breathing apparatus. If not breathing, if breathing is irregular or if respiratory arrest occurs, provide artificial respiration or oxygen by trained personnel. It may be dangerous to the person providing aid to give mouth-to-mouth resuscitation. Get medical attention. If necessary, call a poison centre or physician. If unconscious, place in recovery position and get medical attention immediately. Maintain an open airway. Loosen tight clothing such as a collar, tie, belt or waistband. |
| Ingestion | Get medical attention immediately. Call a poison centre or physician. Wash out mouth with water. Remove dentures if any. Remove victim to fresh air and keep at rest in a position comfortable for breathing. If material has been swallowed and the exposed person is conscious, give small quantities of water to drink. Stop if the exposed person feels sick as vomiting may be dangerous. Aspiration hazard if swallowed. Can enter lungs and cause damage. Do not induce vomiting. If vomiting occurs, the head should be kept low so that vomit does not enter the lungs. Never give anything by mouth to an unconscious person. If unconscious, place in recovery position and get medical attention immediately. Maintain an open airway. Loosen tight clothing such as a collar, tie, belt or waistband. |
| In all cases of doubt, or if symptoms persist seek medical attention. Never give anything by mouth to an unconscious person. If unconscious, place in recovery position and seek medical advice. |  |

| PERSONAL PROTECTIVE EQUIPMENT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| YES | YES | YES/NO | YES | YES | YES/NO |
| Safety glasses or goggles to EN166 | Suitable long sleeved coveralls. |  | Air fed masks EN 270 | PVC or rubber nitrile gloves. EN374 |  |

| HANDLING | STORAGE | TRANSPORT | DISPOSAL |
| --- | --- | --- | --- |
| Put on appropriate personal protective equipment. Do not breathe vapour or mist. Do not swallow. Avoid contact with eyes, skin and clothing. Use only with adequate ventilation. Wear appropriate respirator when ventilation is inadequate. | Do not enter storage areas and confined spaces unless adequately ventilated. Keep in the original container or an approved alternative made from a compatible material, kept tightly closed when not in use. Store and use away from heat, sparks, open flame or any other ignition source. Use explosion-proof electrical (ventilating, lighting and material handling) equipment. Use only non-sparking tools. Take precautionary measures against electrostatic discharges. Empty containers retain product residue and can be hazardous. Do not reuse container |  | Any waste material is classed as hazardous waste, it should only be disposed of through licenced waste handlers and treatment sites. Do not allow unauthorised disposal to the environment. Avoid sources of ignition when handling waste. If operators are exposed to vapours during the disposal process then suitable respiratory protection should be worn. All other PPE should be worn. When handling waste, consideration should be made to the safety precautions applying to handling of the product. |

REQUIRED CONTROL MEASURES

- Users to avoid contact with skin and eyes, and inhalation of vapours/ spray or mist arising from the application of this preparation – Use PPE.

- Respiratory protection if there’s a risk of exposure to high vapour concentrations.

- PVC or rubber gloves.

- Chemical goggles if there is a risk of eye contact.

- Normal work wear.

- Ensure operatives are made aware of and understand the purpose of this assessment and MSDS, plus any health implications of using the product.

FIRE PRECAUTIONS

Suitable: In case of fire, use water spray, foam, dry chemical or CO2. Not suitable: Do not use water jet.

ACCIDENTAL RELEASE/SPILLAGE PROCEDURES

Evacuate surrounding areas. Keep unnecessary and unprotected personnel from entering. Do not touch or walk through spilt material. Shut off all ignition sources. No flares, smoking or flames in hazard area. Avoid breathing vapor or mist. Provide adequate ventilation. Wear appropriate respirator when ventilation is inadequate. Put on appropriate personal protective equipment.

MONITORING REQUIREMENTS AND HEALTH SURVEILLANCE

Annual Health surveillance is place for workers exposed to this product (Lung function and Urine samples)